Carlos A. Soto, Project Manager, GIS Analyst

Curtis D. Klinge, Assistant Manager, GIS Analyst

Bradly D. McDonald, GIS Analyst

Omar A. Rivera Guzman, GIS Analyst

November 2015

GPS Inventory & Impervious Cover Mitigation

Ramon Lucio Park and Baseball Fields

San Marcos, Tx

Prepared for:

Prepared by: KMRS Geospatial Consultants

Table of Contents

1. Introduction	4

1.1 Purpose	4

1.2 Scope	5

2. Data	8

3. Methods	9

3.1 Rio Vista Park Data and GDB Structure Evaluation	11

3.2 Ramon Lucio Park Facilities Inventory - Data Collection	11

3.3 Impervious Cover Calculation and Analysis	12

3.4 Database Merger	13

4. Results	13

4.1 Impervious Cover: Findings and Recommendations	17

4.2 Drainage: Findings and Recommendations	20

5. Conclusion	24

6. References	25

7. Appendix I. Group Participation	25

# 1. Introduction

In 2015, the City of San Marcos (COSM) began implementing a project to utilize Global Positioning System (GPS) technology to carry out a facilities inventory at all parks and recreational areas located on city owned land parcels. One of the first of such projects was carried out during spring 2015 at Rio Vista Park. This pilot project consisted of using a handheld GPS unit to collect coordinate data for a list of features provided by the COSM, and proposing a plan to mitigate impervious cover within park boundaries. The COSM has expressed intent to continue this impervious cover mitigation and reduction project, especially in proximity to the San Marcos River, because scientific research has shown that the sotrmwater runoff created by impervious cover, defined as any and all structures such as rooftops, roadways or parking lots that do not allow water to percolate through (COSM, 2011), carries with it contaminants including oil and grease, organic matter, pesticides, and fertilizer, which can have a negative impact on the water quality of the bodies of water where these pollutants drain into (Flinker 2008). During Fall 2015, KMRS Geospatial Consultants (KMRSGC) was tasked to continue with the implementation of the park inventory process and impervious cover mitigation project at Ramon Lucio Park in San Marcos. The results are expected to be used by the COSM to reduce park maintenance cost, disseminate information to the public, and provide scientific analytics for sound ecological range management.

## 1.1 Purpose

Ramon Lucio Park and Baseball Fields, a 22-acre complex consisting of 5 baseball fields and a river recreation area, is located approximately 1.5 miles downstream of the San Marcos River headwaters in the City of San Marcos, Texas. The complex is an important part of the community. Most of the San Marcos Youth Baseball Softball Association games and tournaments are played on the fields, and the picnic and pavilion areas are favorite place to gather for local residents.

The COSM was very specific in their Request for Proposals about the tasks that the selected group is required to accomplish. First, KMRS Geospatial Consultants analysts will examine the Rio Vista Park geodatabase (GDB) produced by Central Texas Geospatial Consulting (CTGC), determine the spatial and attribute data of park amenities such as grills, gazebos, and parking, as well as field layout and components such as base plugs, dugouts, and illumination. This data will be merged with a different dataset consisting of Rio Vista Park features, creating a GDB that can continue to grow as subsequent inventory projects get carried out at remaining Parks and Recreation properties. This data helps the COSM to make more efficient and informed decisions about future parks and recreation projects. KMRS Geospatial Consultants will then use data collected to evaluate existing impervious cover within park boundaries and propose recommendations for impervious cover mitigation that comply with all applicable legislation and are based on Best Management Practices (BMP) taken from the COSM Green Infrastructure - Low Impact Development (LID) Practices manual (COSM 2011).

## 1.2 Scope

The data collection and impervious cover mitigation portions of this project will be carried out on the parcel of land containing Ramon Lucio Park and Baseball Fields, located at 601 S CM Allen Parkway, in San Marcos, TX. This parcel is bounded by Cheatham Street on the North, C. M. Allen Parkway on the West, South Interstate 35 Frontage Road on the South East side, and the Upper San Marcos River on the North East side. The final products, however, will also include a GDB consisting of Rio Vista Park features merged with the features inventoried and collected at Ramon Lucio Park. Figure 1 is a view of the park, and figure 2 shows a map of the project extent.

Figure 1. Ramon Lucio Park. View from SE looking NW.

Figure 2. Project extent.

# 2. Data

Data utilized for this project was composed of a combination of received data, field-collected data, and downloaded data. From the COSM were received a number of files integral during the process to determine the scope and methods of our project. Provided by the COSM were the following files:

- Ramon Lucio Park Aerial photograph,

- COSM Land Development Code

- Request for Proposal Presentation

- Building, street centerline, edge of pavement, park boundary, and river boundary shapefiles.

- Client Specification files

Also key in this project were the files received from the course instructor, Prof. Yihong Yuan. From the course instructor was received a DVD ROM containing all the files submitted for the Rio Vista Park Project. This disc contained all submitted reports, presentations, and data. The reports and presentations folders contained the presentations and reports prepared for the RFP response, progress update, and final report, the poster prepared for the class project, and the data submitted to the COSM. Data submitted to the COSM consisted of:

- Rio Vista Park ArcMap Document

- San Marcos Parks Data Dictionary

- Rio Vista Park GDB

- 8 images

- 2 Excel spreadsheets: Data Dictionary and Impervious Cover Amounts

- PDF map of GPS collected features

A combination of real-time and lab-based differential correction would be the most effective strategy to produce results with thie highest degree of accuracy possible. Data collected in the field was recorded with a Trimble GeoXT 2005 series high-performance submeter GPS unit with integrated Satellite-Based Augmentation System (SBAS) and multipath technology, checked out on loan from the Department of Geography at Texas State University. In terms of differential correction, in the field real-time SBAS differential correction was enabled on the unit, while in the lab post-processing was accomplished using correction files published by the San Marcos TxDOT Base Station, obtained through Trimble software.

The third portion of the data used in the Ramon Lucio Park project was downloaded from the World Wide Web. This data consisted of documents such as municipal, county and state legislation, peer-reviewed and published scientific research, along with FEMA flood maps and shapefiles, and literature on the topics of LID and Impervious Cover Mitigation, all assembled from a variety of sources published by the public, private, and non-profit sectors. Once sufficient discussion and evaluation informed whether client needs would be able to be addressed using existing data and literature, and the methods available to address those needs were discussed, Best Management Practices (BMPs) for Impervious Cover mitigation guidelines that fit all legal and budgetary requirements were determined. Figure 2 shows LID Options & Considerations applicable to Ramon Lucio Park.

Figure 3. LID Options & Considerations. Source: San Marcos LID Guidance Manual.

# 3. Methods

According to the COSM’s Request For Proposals, the required outcomes for this project were to evaluate previous data collection methodology and file geodatabase architecture, collect data for specific features at Ramon Lucio Park, evaluate existing impervious cover within park boundaries, propose a course of action to mitigate documented impervious cover, and create a merged GDB containing Rio Vista Park and Ramon Lucio Park features. KMRS Geospatial Associates followed a multi-phase process to address these requirements, custom designed for the specific nuances of this particular project. What follows is a discussion of some general aspects of the methods followed, which are described in more detail in subsequent sections, followed by figure 4, showing a summarized visual representation of the process designed and planned by KMRS Geospatial Consultants to meet the requirements specified by the COSM.

The first phase consisted of a review of all Rio Vista Park project data received from the instructor, followed by physical walk-throughs at Ramon Lucio Park. The walk-throughs allowed us to personally get a sense of the physical layout of the park, the park facilities, and to determine all impervious cover existing within park boundaries while allowing for beta testing of the Data Dictionary design before the data collection phase was begun. Once all pieces of impervious cover existing within park boundaries, and the totality of facilities to be mapped were determined through personal site visits and inspections and the data dictionary finalized, the data collection phase could begin.

Data collection was carried out on Oct. 8, 13, 15, 16, and 20. The Trimble GNSS Planning Tool, which allows users to determine optimal satellite geometry on specific dates and times, was used to guide the data collection schedule. Post processing of all field-collected data was carried out within 5 days of data collection date. This ensures correction files used are in as high resolution as possible producing higher quality results. All collected GPS data was exported into shapefile (.shp) format in order to facilitate data dissemination.

Figure 4: Project tasks as planned.

In order to make data generated by or for the COSM easier to share, all the feature classes in Rio_Vista_Park.gdb were also exported to shapefiles. These were saved in a folder named RioVistaShapefiles. Copies of the final shapefiles prepared for the Ramon Lucio Park project were saved in a folder named RamonLucioShapefiles. The folder named MergedShapefiles contains shapefiles, which are the result of merging feature classes appearing in more than one COSM park (for example, since both Rio Vista and Ramon Lucio Parks both contained benches, both shapefiles were merged and the result saved in the MergedShapefiles folder as Benches_all.shp and imported to SanMarcosParks.gdb).

### 3.1 Rio Vista Park Data and GDB Structure Evaluation

The first task required a methodical analysis of all received Rio Vista Park project files. All project files were examined for completeness and functionality. The specific logging options defined in the Trimble data dictionary file were carefully inspected, and its structure compared to the file titled DATADICTIONAR.xlsx. Also, the overall structure of the GDB was evaluated to determine if any changes could help the COSM make more efficient use of its GIS data. Additionally, the methodology implemented by the Rio Vista Park project team to carry out their project at was evaluated for clarity and quality of subsequent results. The deliverables produced for this project specify changes that our group determined would produce more accurate data for the COSM, and help subsequent groups accomplish equal or better results.

### 3.2 Ramon Lucio Park Facilities Inventory - Data Collection

In terms of Logging Settings, Accuracy Based Logging was not used due to the fact that trial data-collection runs suggested that the picnic tables located in proximity to the pedestrian bridge across the river might be difficult to map with this specification, on account of the GPS signal interference caused by the dense foliage of the large pecan trees above resulting in unacceptable accuracies. It was determined that, since the COSM expressed the intent of removing these extents of impervious cover and, consequently, the requirements for accuracy could be low for these specific features, unacceptable positions for other features could be deleted during post-processing. For point features, a minimum of 30 positions was collected per point feature. Lin and Polygon features were collected using a logging interval of 5 seconds.  Real-time Satellite Based Augmentation System (SBAS) correction signals were used for in-the-field differential correction while collecting GPS data for all features, which provides another dimension of accuracy producing results of the highest quality.

Data was post processed using Code Range measurements acquired from TxDOT Continuously Operating Reference Stations (CORS) located in San Marcos for differential correction. Once raw data was differentially corrected, positions for individual points were filtered according to the following criteria:

- Positions with Horizontal Accuracy between 0 and 40” were kept.

- Positions with a Height Dilution of Precision (HDOP) of 10 or more were deleted.

- Positions with a Positional Dilution of Precision (PDOP) of 10 or more were deleted.

Once positions were filtered according to these criteria, and accuracies of under 19” were achieved for 50% or more positions & accuracies of under 28” were achieved for 90% or more positions, data was exported into shapefile format. The shapefiles are in the State Plane – Texas South Central FIPS 4202 coordinate system, with horizontal units in feet, according to client specifications. Final accuracy of all data resulted in 68% accuracies within 20”.

Shapefile rectification was accomplished using high-resolution aerial imagery provided by the COSM at the beginning of the project. The aerial image was georeferenced using a combination of 4 points collected specifically for georeferencing the image, and some of the points collected for the project. The points collected for georeferencing were located more than ¼ mile away from the nearest park boundary, and were also selected for ease of identification in aerial imagery. Once a best fit was accomplished for the aerial image, line and polygon features were georectified to accurately reflect relative feature location as they appear on the image. Most of the features in the Fencing layer were created by tracing the georeferenced aerial image.

### 3.3 Impervious Cover Calculation and Analysis

Impervious Cover totals were calculated in accordance with San Marcos City Code, using the rectified Impervious Cover layer polygons created from the code-corrected GPS data collected in the field. Total areas for each of the impervious cover polygons existing within park boundaries were summed, and the results divided by the area of the Park Boundary polygon (provided by the COSM). In order to determine areas in the park falling within Water Quality, and Buffer Zones, as defined in Chapter 5, Article 3, a Buffer Analysis was performed to find the areas within 100 and 200 feet of the bank of the San Marcos River.

The Low Impact Development (LID) strategies recommended in section 4.1 are based on the information found in the City of San Marcos Green Infrastructure – Low Impact Development Practices Guidebook (COSM 2011). Recommended changes include a number of structural and non-structural LID practices and sustainable stormwater management techniques from a list developed by the COSM to comply with all applicable regulations and policies. As specified in Section 5.1.2.4 – Impervious Cover Limitations, existing impervious cover may be replaced, subject to standards in article 4 of Chapter 5, but not increased. The implications for Ramon Lucio Park are that impervious cover, if existing within these zones, will be required to be removed and re-located to the parts of the property farthest away from the boundaries of the San Marcos River.

### 3.4 Database Merger

Merging of two GDBs can be accomplished using a multitude of different methods, which vary according to the needs of each specific project and user. In this case, it is useful for the COSM to know the geographical location of the feature(s) being examined, as well as within what park or property the features being examined are located. The different methods available to accomplish this distinction in a relational database were discussed and with the input of Pete Binion, GIS/Transit Analyst with the Community Services/ Engineering department of the COSM, it was determined that attribution should be added to the Rio Vista Park and Ramon Lucio Park datasets. To this end, a text attribute was added to each shapefile in the Rio Vista and Ramon Lucio park folders. This attribute was named “PARK_NAME”, of type “string” with a length of 50 characters, and populated with values “Rio Vista Park” for Rio Vista Park, and “Ramon Lucio Park” for Ramon Lucio Park. This way, users will have available two different methods for distinguishing facilities for each field class: search by geographical location or select by attributes.

# 4. Results

When the COSM selected KMRS Geospatial Consultants to conduct this project, they were specific about the tasks required. Our first task, examining the Rio Vista Park project files, began with the final data dictionary. It is important to note that there were three different files labeled “Data Dictionary” found within the received data. One was in Trimble Data Dictionary Format (named “San Marcos Parks Data Dictionary.ddf ”), another was a text file (named “San Marcos Parks Data Dictionary.txt”) and the other was a Microsoft Excel spreadsheet (named “DATADICTIONARY.xlsx”). Close scrutiny of the data dictionary used for the Rio Vista Park GDB revealed that only 1 position was recorded per point feature (fig. 5a). Our group also found that two of the individual feature classes appearing in the Rio Vista Park GDB (point features named Valves and Sprinklers) could be combined into a single layer named Irrigation, differentiated by an attribute named TYPE, resulting in a more efficient GDB structure (fig. 5b). Seemingly insignificant changes as these may not make much difference on a small GDB with a minor extent, but on a GDB with a greater number of features extending over a larger area, it is imperative to design and build data structures as efficiently as possible.

Data collection at Ramon Lucio Park, paradoxically enough, required more trips to the site than initially had been predicted, but was completed ahead of schedule. This is because, during the first two days of data collection, our group realized the immensity of the task and faced it, head on. We quickly coordinated our schedules to prioritize this phase of the process and completed data collection in 3 weeks. Task completion required coordination with multiple individuals from different departments of the COSM. When presented with questions on topics as diverse as the database structure and file specifications of the project, to difficulty in gaining access to the baseball fields at Ramon Lucio Park, everybody we interacted with working for the COSM impressed our group with their degree of professionalism and helpfulness.

Final post-processing and georectification of line and polygon shapefiles consumed the most time of the project, and resulted in a deviation of the original task workflow as illustrated in figure 4. As a result, the merging of the Rio Vista Park and Ramon Lucio Park geodatabases was moved to later in the workflow. This change in the process order did not affect any findings or results. The most labor-intensive component of this project, on the other hand, turned out to be the data collection component. Fortunately, the analysis requirements of this project were minimal.

Figure 5a. (left) shows San Marcos Parks.ddf. Note, on the lower right corner of 5a, the number of positions recorded for the point feature class labeled ADA, highlighted in blue in the Feature list. Figure 5b. (right) shows the .xcl file containing separate feature classes for irrigation sprinklers and irrigation valve boxes.

The only analysis necessary was to create two buffers around the banks of the San Marcos River in order to determine whether the floodplain, the Water Quality Zone (100 ft buffer), or the Buffer Zone (200ft buffer) occupied more area. The result is that Ramon Lucio Park has areas that fall within a Water Quality Zone, as well as areas that fall within a Buffer Zone. Impervious cover in Water Quality Zones is prohibited, but existing cover may be replaced. Meanwhile,in Buffer Zones, impervious cover is limited by the gradient of the land to be developed, and the recommendations in section 4.1 reflect the requirements specified in section 5.1.2.4 of San Marcos Municipal Code. Water Quality, Buffer, and Floodplain zones are shown in figure 5. In order to determine how much impervious cover fell within each zone, a polygon for the portion of the 200’ buffer zone not covered by the 100’ buffer zone, and a polygon for the 100’ buffer zone (both within park boundaries) were created. Then, the Impervious Cover polygon layer was clipped for each of the 3 zones (100’, 200’, and beyond).

Figure 6. Ramon Lucio Park Flood, Water Quality, and Buffer zones.

When the baseball field base location data was reviewed, we found that the base distances needed to be re-measured. When using the measure tool to determine base distances, it was found that most bases were not the required distance apart. Some were closer, while other bases were farther. Since the discrepancy might be the result of GPS error, we recommend that the youth baseball league physically measure and review the base distances before the beginning of the next coming season. Another issue that was found in the baseball fields at Ramon Lucio Park were the cracks that developed as a result of the drought that affected the area this summer. The cracks had been filled with sand by COSM facilities personnel, as was observed while data for this project was collected. When the site was inspected after the rain on Nov 17, the sand was found to have settled into the cracks, making the same cracks (albeit not as deep now) to reappear on field surfaces.

### 4.1 Impervious Cover: Findings and Recommendations

Ramon Lucio Park was designed and built in the 1980’s. The design was simple, practical, and functional, prioritizing the needs of the local and visiting citizens who would spend their time there. As time progressed and the city grew, city development policies changed to reflect the growing trends in urban planning that other cutting-edge, growing cities across the globe began implementing. The COSM administrators shifted towards policies that center around the improvement of the quality of life of the city through the conservation of natural resources and improvement of urban green spaces, with special emphasis placed on the San Marcos River Corridor. As we look at Ramon Lucio Park’s design with a fresh set of eyes, unfortunately, we do not like what we see.

The total amount of impervious cover existing within Ramon Lucio Park was found to be 183,267 sq ft (4.21 acres), or 22.6% of existing park area (810,390 sq ft or 18.6 acres). Within 100ft of the banks of the San Marcos River exist 7,294.79 sq ft of impervious cover, or 5% of this zone’s area. Within 200ft of the banks there were found to be 16,939.64 sq ft of impervious cover, or 12% of this zone’s area. In the area lying within park boundaries but beyond the 200ft buffer line, 159,032.58 sq ft of impervious cover were found, or 20% of this area. The impervious cover within the 100ft buffer will need to be removed in order to continue the Bank Restoration Project that has been carried out at other COSM parks. Appendix I shows a map of impervious cover within each buffer zone.

The COSM LID Guidance Manual recommends a number of non-structural LID practices such as conservation design, preservation of existing resources such as healthy riparian buffers, or disconnection of impervious surfaces, that we recommend should also be implemented at Ramon Lucio Park. The bank restoration project should continue as planned in order to continue reducing the number of pollutants that get carried by stormwater runoff into the main channel, thus improving the water quality of the San Marcos River. Riparian areas in sections of the San Marcos River composed of gravelly sediments deposited by the river have substantial infiltration rates, which are excellent at reducing runoff (through infiltration action) and eliminating surface discharge.

In terms of structural changes that implement LID practices, a number of interventions are required. First, a concerning number of 14ft by 14 ft impervious picnic table pads of concrete and peagravel construction were found to be clustered within the Water Quality Zone, in very close proximity to the banks of the San Marcos River. Collectively, the pads add up to square feet and need to be eliminated. If the picnic tables, which are a favorite gathering place for local families as our group saw on multiple occasions, are to remain in the park, our recommendation is to build pads out of compacted gravel, gabions, or permeable pavement such as grasscrete.

The largest single expanse of impervious cover existing within Ramon Lucio Park boundaries is the asphalt-surfaced parking area near the pavilion that contains the 4 double-length picnic tables. It is in very close proximity to the riverbank (a portion is actually within the buffer zone–see fig. 6) and drains into a retention feature (fig. 7). The total area covered by the polygon in the GDB representing this single, unbroken extent of impervious cover is (NEED TOTAL). This parking lot is also located in some places, within 60 feet of the San Marcos River boundary. In order to bring the property up to compliance with contemporary LID standards, it is recommended that this extent of impervious cover be removed and portions within the Buffer zone restored according to River Bank Restoration Project guidelines, and remaining portions to be converted into either pervious parking area or runoff retention areas, as space allows. It is further recommended that the principal parking area be moved to the area along the Interstate 35 frontage road, currently a grassy swale used for drainage. Pavers, compacted gravel, grasscrete, and other low-cost options are available to permeably surface this new parking area with.

Due to the large number of people that use these fields during the youth baseball season, it is imperative to make the most efficient use of the area designated for parking as possible, resulting in the necessity for more efficient parking lot layouts to be considered. If possible, developers should adapt the layout and angle of parking stalls in any new parking area, and include a certain number of compact car only spaces, to achieve the greatest lot capacity. Swales, vegetated filter strips/riparian buffers, bioretention areas, and infiltration systems are some structural BMPs recommended by the EPA to use in green parking lot design (EPA 2008).

Figure 7. Retention feature.

### 4.2 Drainage: Findings and Recommendations

The design of the stormwater runoff drainage system is another element of concern for Ramon Lucio Park in terms of its contribution to the quality of the San Marcos River. The retention pond collecting runoff created by the large, asphalt surfaced parking lot mentioned previously is fitted with an overflow consisting of a grated drain that feeds into a culvert. This culvert leads to an outfall located a few feet from the banks of the San Marcos River (fig. 8). Although fig. 7 clearly shows water levels have not reached the overflow leading to this culvert, a small flow of water was nonetheless observed flowing down the outfall and making its way into the San Marcos River, product of possible breaches of the culvert wall resulting in water percolation accumulating into a discharge.

When Ramon Lucio Park was visually inspected after a rainfall, large puddles were found accumulating in the parking area (fig. 9a, 9b). It was also found that the water draining from the parking area into the retention feature was contaminated with what visual inspection seemed to identify as residual vehicular oil accumulation (fig. 9a, 9c). Since these particular retention features were not designed to filter chemical contaminants from stormwater runoff, it is recommended they be converted to Bioretention Features or Rain Gardens, which are landscaping features that retain stormwater runoff in addition to providing a soil and plant-based filtration device that through a variety of physical, biological, and chemical treatment processes remove pollutants as well (COSM 2011). A vegetated filter strip, such as is being created by the Bank Restoration Project in other areas of the Upper San Marcos River, is an excellent addition that will contribute to chemical pollutant removal by promoting infiltration of any remaining surface runoff that does not percolate through.

Figure 8. Outfall draining into river. In the background is the southbound I-35 frontage road bridge. The San Marcos River is located between outfall and bridge.

Figure 9a. (top left) and 9b. (top right) show opposing views of aspahalt-surfaced parking feature. Figure 9c shows a closeup view of figure 9a, note the contaminants including oil, trash, organic matter and suspended sediment.

# 5. Conclusion

KMRS Geospatial Consultants successfully completed a facilities inventory at Ramon Lucio Park and Baseball Fields and proposed an impervious cover mitigation plan for the site. The challenges faced when carrying out this project helped us grow professionally as solutions were developed for each unique task. Building upon the work of others, existing data acquisition methods were improved while an expandable geodatabase that can be updated according to the COSM’s growing needs was created. The data produced as a result of these projects can help the COSM in reaching their objectives of reducing park maintenance cost, the dissemination of information to the public, and providing scientific analytics for sound ecological range management by providing more accurate facts to inform future data-driven decisions.

# 6. References

Flinker, Peter; 2008. The Need To Reduce Impervious Cover to Prevent Flooding and Protect Water Quality. Rhode Island Department of Environmental Management, 2008, revised 2010.

City of San Marcos; 2011. San Marcos: Green Infrastructure – LID Practices & LID Guidance Manual. November 2011.

City Of San Marcos; 2014. Stormwater Technical Manual. February 2014.

Environmental Protection Agency, US Green Parking Lot Resource Guide. February, 2008.

Trimble GNSS Planning Tool Online: http://www.trimble.com/GNSSPlanningOnline

# 7. Appendix I. Impervious Cover within Park Zones

# 8. Appendix II. Group Participation

Curtis

Proposal & Presentation: Formatting slides, Flowchart creation, Logo, Timeline, and Budget

Progress Report & Presentation: Prepared presentation slide wireframes.

Final Report & Presentation: Contributed to Data section, prepared wireframe.

Data Dictionary: Added point features.

Data Collection: Collected data on Oct.: 5, 12, 19, and Nov. 2.

Shapefile Rectification: Rectified line and polygon features (fences, paths, impervious cover).

Impervious Cover Analysis: Background research.

Mitigation Recommendations: Background research.

Geodatabase: Organization and simplification.

Metadata: Helped Brad with filling out Metadata sections, particularly Domain for each attribute of each feature.

Poster: Helped create scope map and offered help for map layout.

Brad

Proposal & Presentation: Contributed to implications, final deliverables, and flow chart.

Progress Report & Presentation: Helped edit report, inventory data collection, Challenges, and GPS data information.

Final Report & Presentation: Helped proofread final report, presented on data and major concerns with impervious cover.

Data Dictionary: Added line features.

Data Collection: Collected data on Oct.: 5, 6, 7, 12, 13, 14, 15, 19, 20, and Nov. 2.

Shapefile Rectification: Helped edit park polygon and line features such as the baseball fields, impervious cover, buildings and amenities throughout the park.

Impervious Cover Analysis: Background research.

Mitigation Recommendations: Background research.

Metadata: Contributed to doing half of the metadata.

Poster: Contributed to editing and creating final maps.

Omar

Proposal & Presentation: Software, Methodology, and References section for both Proposal and presentation.

Progress Report & Presentation: Helped edit report, inventory data collection.

Final Report & Presentation:

Data Dictionary: Added area features in the field.

Data Collection: Collected data on Oct.: 5, 7, 12, 14, 19, and Nov. 2.

Postprocessing: Used GPS Pathfinder to help Carlos with postprocessing.

Shapefile Rectification: Used Advanced Editing to simplify and “Clean Up” features collected in the field.

Impervious Cover Analysis: Background research.

Mitigation Recommendations: Background research.

Geodatabase: Organization.

Metadata: First half of list of features were completed.

Poster: Prepared entire poster layout and contributed content (maps).

Carlos

Proposal & Presentation: Final writeup, re-wording and editing.

Progress Report & Presentation: Wrote, edited, and submitted.

Final Report & Presentation: Wrote, edited, and submitted.

Data Dictionary: Evaluated previous and specified settings.

Data Collection: Collected data on Oct.: 5, 6, 7, 12, 13, 14, 15, 19, 20, 27, 29, and Nov. 2.

Postprocessing: Differential corrected all GPS data.

Shapefile Rectification: exported rectified shapefiles.

Impervious Cover Analysis: Determined zones, quantified impervious cover.

Mitigation Recommendations: Determined BMP.

Geodatabase: Created GDB and imported shapefiles.

Metadata: Copied metadata into final report.

Poster: Wrote text and contributed images.

# 9. Appendix III. Metadata

# Ramon Lucio Park Features File Geodatabase

Tags
Ramon Lucio Park, Features, Geodatabase, City of San Marcos, Texas State University

Summary

The purpose of this Geodatabase is to provide the City of San Marcos with an inventory of data that was collected in the field. The inventory will help the COSM maintain and expand on the data, which will aid users to make more efficient and informed decisions about future park and recreation projects.

Description

In 2015, the City of San Marcos (COSM) began implementing a project to utilize Global Positioning System (GPS) technology to carry out a facilities and feature inventory at all parks and recreational areas located on city owned land parcels. KMRS Geospatial Consultants determined the spatial and attribute data of park amenities such as grills, pavilians, fields, picnic areas, buildings, paths, parking lots, illumination poles, irrigation, and the amount of impervious cover in the park.

Credits

Carlos Soto; Project Manager, Geodatabase Administrator

Curtis Klinge; Assistant Project Manager

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.936197 | East | -97.927882 |
| --- | --- | --- | --- |
| North | 29.879209 | South | 29.872157 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Citation  ▼►

Title Ramon Lucio Park Features File Geodatabase

Creation date 2015-11-22 00:00:00

Publication date 2015-11-30 00:00:00

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Carlos Soto

Organization's name KMRS Geospatial Consultants

Contact's position Project Manager

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Credits

Carlos Soto; Project Manager, Geodatabase Administrator

Curtis Klinge; Assistant Project Manager

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Geodatabase

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

West longitude -97.936197

East longitude -97.927882

South latitude 29.872157

North latitude 29.879209

Extent contains the resource Yes

Temporal extent

Beginning date 2015-11-24 00:00:00

Ending date 2015-11-30 00:00:00

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Metadata Details  ▼►

Metadata character set  utf8 - 8 bit UCS Transfer Format

Scope of the data described by the metadata  dataset

Last update 2015-11-25

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata ItemDescription

Created in ArcGIS for the item 2015-11-24 16:53:25

Last modified in ArcGIS for the item 2015-11-30 85:75:50

Automatic updates

Have been performed No

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position GIS Analyst

Contact's role  originator

Contact information  ▼►

Phone

Voice (512) 245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Metadata contact

Individual's name Carlos Soto

Organization's name KMRS Geospatial Consultants

Contact's position Project Manager, GIS Analyst, Geodatabase Administrator

Contact's role  originator

Contact information  ▼►

Phone

Voice (512) 245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address cas349@txstate.edu

Hide Contact information ▲

Metadata contact

Individual's name Curtis Klinge

Organization's name KMRS Geospatial Consultants

Contact's position Assistant Project Manager, GIS Analyst

Contact's role  originator

Contact information  ▼►

Phone

Voice (512) 245-2111

Address

Type postal

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address cdk38@txstate.edu

Hide Contact information ▲

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's position GIS Analyst

Contact's role  originator

Contact information  ▼►

Phone

Voice (512) 245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address oar8@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## Metadata Maintenance  ▼►

Maintenance

Update frequency  annually

Hide Metadata Maintenance ▲

## Thumbnail and Enclosures  ▼►

Thumbnail

Thumbnail type  JPG

Hide Thumbnail and Enclosures ▲

# ADA

File Geodatabase Feature Class

Thumbnail Not Available

Tags
ADA, ADA Parking, ADA Compliant

Summary

To show locations that meet ADA standards for parking and ramps in Ramon Lucio Park.

Description

ADA compliant Parking Features and ADA compliant Ramp Features that were collected by GPS.

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.935126 | East | -97.932401 |
| --- | --- | --- | --- |
| North | 29.877211 | South | 29.873413 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

Themes or categories of the resource  location

* Content type  Downloadable Data

Export to FGDC CSDGM XML format as Resource Description No

Theme keywords  ADA, Ramps, Parking

Hide Topics and Keywords ▲

## Citation  ▼►

* Title ADA

Creation date 2015-10-13 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map, digital document

FGDC geospatial presentation format  map, document

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Carlos Soto

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Fax 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

Country US

e-mail address cas349@txstate.edu

Hide Contact information ▲

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name ADA

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.935126

* East longitude -97.932401

* North latitude 29.877211

* South latitude 29.873413

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-13 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2305982.555869

* East longitude 2306833.778248

* South latitude 13866667.001416

* North latitude 13868040.430756

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name ADA

* Object type  point

* Object count 7

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name ADA

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 7

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Data Quality  ▼►

Scope of quality information  ▼►

Resource level  attribute

Hide Scope of quality information ▲

Hide Data Quality ▲

## Geoprocessing history  ▼►

Process

Process name

Date 2015-11-22 15:03:54

Tool location c:\program files (x86)\arcgis\desktop10.3\ArcToolbox\Toolboxes\Data Management Tools.tbx\CalculateField

Command issued

CalculateField ADA_Ramps%%% TYPE "[TYPE] = Ramp" VB #

Include in lineage when exporting metadata No

Hide Geoprocessing history ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object ADA  ▼►

* Type Feature Class

* Row count 7

Definition

ADA parking features and ADA ramp features

Definition source

Trimble GeoXT 2005 series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-99

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-999999999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Position

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-9999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Point_ID ▲

Field TYPE  ▼►

* Alias TYPE

* Data type String

* Width 50

* Precision 0

* Scale 0

Field description

Distinguishes whether a ramp or parking

Description source

Trimble GeoXT 2005 Series

List of values

Value ADA

Description ADA

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field TYPE ▲

Hide Details for object ADA ▲

Hide Fields ▲

## Metadata Details  ▼►

Metadata language English (UNITED STATES)

Metadata character set  utf8 - 8 bit UCS Transfer Format

Scope of the data described by the metadata  dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-22 15:03:54

Last modified in ArcGIS for the item 2015-11-30 13:55:28

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:55:28

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Carlos Soto

Organization's name KMRS Geospatial Associates

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Fax 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

Country US

e-mail address cas349@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## Metadata Maintenance  ▼►

Maintenance

Update frequency  annually

Hide Metadata Maintenance ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label ADA

Entity Type Definition

ADA parking features and ADA ramp features

Entity Type Definition Source Trimble GeoXT 2005 series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Position

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label TYPE

Attribute Definition

Distinguishes whether a ramp or parking

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value ADA

Enumerated Domain Value Definition

ADA

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# Bases

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Bases, Baseball Field Bases, Facilities inventory, Baseball, Bases

Summary

To show the geographic locations of bases for the baseball fields.

Description

All bases are recorded in single file type. There are no differences between the different baseball fields.

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934749 | East | -97.933105 |
| --- | --- | --- | --- |
| North | 29.876515 | South | 29.874672 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

Themes or categories of the resource  environment, inlandWaters, location, planningCadastre, society

* Content type  Downloadable Data

Export to FGDC CSDGM XML format as Resource Description No

Theme keywords  Bases, Baseball Field Bases

Theme keywords  Facilities inventory, Baseball, Bases

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Bases

Creation date 2015-10-12 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  digital document, digital map

FGDC geospatial presentation format  document

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. RIvera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Bases

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934749

* East longitude -97.933105

* North latitude 29.876515

* South latitude 29.874672

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306098.076308

* East longitude 2306612.936498

* South latitude 13867122.616974

* North latitude 13867788.293793

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Bases

* Object type  point

* Object count 28

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Bases

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 28

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Bases  ▼►

* Type Feature Class

* Row count 28

Definition

Single TYPE represneting Baseball Field Base Location

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Point_ID ▲

Hide Details for object Bases ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:22

Last modified in ArcGIS for the item 2015-11-30 13:55:52

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:55:52

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Tx

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Bases

Entity Type Definition

Single TYPE represneting Baseball Field Base Location

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# BBQ_Grill

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Barbeque Grill

Summary

Barbeque Grill Location

Description

Barbeque Grill

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.932211 | East | -97.931720 |
| --- | --- | --- | --- |
| North | 29.876496 | South | 29.876018 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title BBQ_Grill

Creation date 2015-10-12 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name BBQ_Grill

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.932211

* East longitude -97.931720

* North latitude 29.876496

* South latitude 29.876018

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-11-30 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306897.685601

* East longitude 2307051.989754

* South latitude 13867616.311525

* North latitude 13867788.485065

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name BBQ_Grill

* Object type  point

* Object count 7

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name BBQ_Grill

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 7

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object BBQ_Grill  ▼►

* Type Feature Class

* Row count 7

Definition

Barbeque Grill

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Idnetifier

Description source

Trimble GeoXT 2005 Series GPS Unit

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Point_ID ▲

Hide Details for object BBQ_Grill ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:25

Last modified in ArcGIS for the item 2015-11-30 13:56:11

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:56:11

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label BBQ_Grill

Entity Type Definition

Barbeque Grill

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Idnetifier

Attribute Definition Source Trimble GeoXT 2005 Series GPS Unit

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# Bike_Rack

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Bike Rack

Summary

Bike Rack Location

Description

Bike Rack

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.933987 | East | -97.933986 |
| --- | --- | --- | --- |
| North | 29.874450 | South | 29.874449 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Bike_Rack

Creation date 2015-10-07 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Bike_Rack

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

West longitude -97.933987

* East longitude -97.933986

North latitude 29.874450

South latitude 29.874449

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306340.539734

* East longitude 2306340.539734

* South latitude 13867039.358283

* North latitude 13867039.358283

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Bike_Rack

* Object type  point

* Object count 1

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Bike_Rack

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 1

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Bike_Rack  ▼►

* Type Feature Class

* Row count 1

Definition

Bike Rack

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identfier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Point_ID ▲

Hide Details for object Bike_Rack ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:28

Last modified in ArcGIS for the item 2015-11-30 13:57:05

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:57:05

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Bike_Rack

Entity Type Definition

Bike Rack

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identfier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# Bleachers

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Bleachers

Summary

Location of bleachers around baseball fields

Description

Bleachers

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934786 | East | -97.933432 |
| --- | --- | --- | --- |
| North | 29.876289 | South | 29.874960 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Bleachers

Creation date 2015-10-12 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Bleachers

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934786

* East longitude -97.933432

* North latitude 29.876289

* South latitude 29.874960

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306085.204286

* East longitude 2306509.936376

* South latitude 13867226.596425

* North latitude 13867706.050191

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Bleachers

* Object type  point

* Object count 16

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Bleachers

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 16

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Bleachers  ▼►

* Type Feature Class

* Row count 16

Definition

Bleachers

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Point_ID ▲

Hide Details for object Bleachers ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:32

Last modified in ArcGIS for the item 2015-11-30 13:57:27

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:57:27

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Bleachers

Entity Type Definition

Bleachers

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# Building

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Building

Summary

Building Location

Description

Buildings within the park

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934543 | East | -97.933637 |
| --- | --- | --- | --- |
| North | 29.875522 | South | 29.875121 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Building

Creation date 2015-10-14 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Building

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934543

* East longitude -97.933637

* North latitude 29.875522

* South latitude 29.875121

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306161.669028

* East longitude 2306447.715044

* South latitude 13867284.603855

* North latitude 13867427.797794

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Building

* Object type  composite

* Object count 2

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Building

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 2

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Building  ▼►

* Type Feature Class

* Row count 2

Definition

Building

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field TYPE  ▼►

* Alias TYPE

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Concesions Building, Maintenance Shack, Pavillion.

Description source

Trimble GeoXT 2005 Series

List of values

Value Concessions Building, Maintenance Shack, Pavillion.

Description Building Structures

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field TYPE ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999999

Description Area

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Idnetifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-99999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Building ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:42

Last modified in ArcGIS for the item 2015-11-30 13:58:10

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:58:10

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Building

Entity Type Definition

Building

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label TYPE

Attribute Definition

Concesions Building, Maintenance Shack, Pavillion.

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Concessions Building, Maintenance Shack, Pavillion.

Enumerated Domain Value Definition

Building Structures

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999999

Enumerated Domain Value Definition

Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Idnetifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# Drainage_Area

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Drainage

Summary

Drainage Location

Description

Drainage features within the Park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934684 | East | -97.932420 |
| --- | --- | --- | --- |
| North | 29.875759 | South | 29.873412 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Drainage_Area

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Drainage_Area

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934684

* East longitude -97.932420

* North latitude 29.875759

* South latitude 29.873412

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306122.711757

* East longitude 2306832.524314

* South latitude 13866666.615262

* North latitude 13867513.454775

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Drainage_Area

* Object type  composite

* Object count 2

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Drainage_Area

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 2

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Drainage_Area  ▼►

* Type Feature Class

* Row count 2

Definition

Drainage

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Comment  ▼►

* Alias Comment

* Data type String

* Width 32

* Precision 0

* Scale 0

Field description

Drainage features within Park

Description source

Trimble GeoXT 2005 Series

List of values

Value drainage

Description comment

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Comment ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description GNSS

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Position

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Area

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-999

Description Shape and length

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-999

Description Shape/Area

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Drainage_Area ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-22 16:18:52

Last modified in ArcGIS for the item 2015-11-30 13:58:35

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:58:35

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Drainage_Area

Entity Type Definition

Drainage

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Comment

Attribute Definition

Drainage features within Park

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value drainage

Enumerated Domain Value Definition

comment

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

GNSS

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Position

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Shape and length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Shape/Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Hide Entities and Attributes ▲

# Drainage_Grates

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Drainage Grates

Summary

Drainage Grates Location

Description

Drainage Grates within park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934544 | East | -97.932563 |
| --- | --- | --- | --- |
| North | 29.875685 | South | 29.873715 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Drainage_Grates

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Drainage_Grates

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934544

* East longitude -97.932563

* North latitude 29.875685

* South latitude 29.873715

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306165.977091

* East longitude 2306787.554588

* South latitude 13866776.385383

* North latitude 13867487.051285

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Drainage_Grates

* Object type  point

* Object count 2

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Drainage_Grates

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 2

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Drainage_Grates  ▼►

* Type Feature Class

* Row count 2

Definition

Drainage Grates

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field TYPE  ▼►

* Alias TYPE

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Drainage Grate

Description source

Trimble GeoXT 2005 Series

List of values

Value Drainage

Description Drainage Grate

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field TYPE ▲

Field DRAIN  ▼►

* Alias DRAIN

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Drainage Grate

Description source

Trimble GeoXT 2005 Series

List of values

Value Drain

Description Drain

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field DRAIN ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Position

Enumerated domain value definition source Trimble GeoXT 2005 Series GPS Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Unique Identifier

Hide Field Point_ID ▲

Hide Details for object Drainage_Grates ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:35

Last modified in ArcGIS for the item 2015-11-30 13:58:51

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:58:51

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Drainage_Grates

Entity Type Definition

Drainage Grates

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label TYPE

Attribute Definition

Drainage Grate

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Drainage

Enumerated Domain Value Definition

Drainage Grate

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label DRAIN

Attribute Definition

Drainage Grate

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Drain

Enumerated Domain Value Definition

Drain

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Position

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series GPS Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Unique Identifier

Hide Entities and Attributes ▲

# Dugouts

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Dugouts

Summary

Dugouts Location

Description

Location of dugouts connected to the baseball fields

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934842 | East | -97.933249 |
| --- | --- | --- | --- |
| North | 29.876446 | South | 29.874792 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Dugouts

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Dugouts

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934842

* East longitude -97.933249

* North latitude 29.876446

* South latitude 29.874792

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306068.088507

* East longitude 2306567.486458

* South latitude 13867166.053567

* North latitude 13867762.985116

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Dugouts

* Object type  composite

* Object count 10

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Dugouts

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 10

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Dugouts  ▼►

* Type Feature Class

* Row count 10

Definition

Dugouts

Definition source

Trimble GeoXT 2005 Series Unit

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Definition

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-99999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Dugouts ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:45

Last modified in ArcGIS for the item 2015-11-30 13:59:11

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:59:11

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Dugouts

Entity Type Definition

Dugouts

Entity Type Definition Source Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Definition

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Electric

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Electric

Summary

Location of main electric box

Description

Main Electric Box

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.933644 | East | -97.933582 |
| --- | --- | --- | --- |
| North | 29.875098 | South | 29.875051 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Electric

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Electric

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.933644

* East longitude -97.933582

* North latitude 29.875098

* South latitude 29.875051

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306446.811831

* East longitude 2306466.289810

* South latitude 13867259.043539

* North latitude 13867275.912271

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Electric

* Object type  composite

* Object count 1

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Electric

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 1

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Electric  ▼►

* Type Feature Class

* Row count 1

Definition

Main Electric Box

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-20

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Electric ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:48

Last modified in ArcGIS for the item 2015-11-30 13:59:34

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:59:34

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Electric

Entity Type Definition

Main Electric Box

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Electric_Boxes

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Electric Boxes

Summary

Location of secondary electric boxes

Description

secondary electric boxes on the side of each baseball field

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934759 | East | -97.933808 |
| --- | --- | --- | --- |
| North | 29.877175 | South | 29.875112 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Electric_Boxes

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Electric_Boxes

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934759

* East longitude -97.933808

* North latitude 29.877175

* South latitude 29.875112

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306093.207223

* East longitude 2306388.000597

* South latitude 13867280.450320

* North latitude 13868028.175531

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Electric_Boxes

* Object type  point

* Object count 4

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Electric_Boxes

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 4

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Electric_Boxes  ▼►

* Type Feature Class

* Row count 4

Definition

Secondary Electric Boxes

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Electric_Boxes ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:38

Last modified in ArcGIS for the item 2015-11-30 13:59:50

Automatic updates

Have been performed Yes

Last update 2015-11-30 13:59:50

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San MArcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Electric_Boxes

Entity Type Definition

Secondary Electric Boxes

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Emergency

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Emergency

Summary

Location of emergency siren

Description

Emergency Siren

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.932967 | East | -97.932966 |
| --- | --- | --- | --- |
| North | 29.875894 | South | 29.875893 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Emergency

Creation date 2015-10-14 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Emergency

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

West longitude -97.932967

* East longitude -97.932966

North latitude 29.875894

* South latitude 29.875893

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-23 00:00:00

Extent in the item's coordinate system

* West longitude 2306658.900973

* East longitude 2306658.900973

* South latitude 13867567.076387

* North latitude 13867567.076387

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Emergency

* Object type  point

* Object count 1

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Emergency

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 1

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Emergency  ▼►

* Type Feature Class

* Row count 1

Definition

Emergency Siren

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Emergency ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:40

Last modified in ArcGIS for the item 2015-11-30 14:00:20

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:00:20

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Emergency

Entity Type Definition

Emergency Siren

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Fencing

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Fence

Summary

Location of fence around and within the park boundaries

Description

Fencing around and within park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934906 | East | -97.931787 |
| --- | --- | --- | --- |
| North | 29.876852 | South | 29.874355 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Fencing

Creation date 2015-10-12 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Fencing

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934906

* East longitude -97.931787

* North latitude 29.876852

* South latitude 29.874355

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306049.327061

* East longitude 2307029.579694

* South latitude 13867011.275990

* North latitude 13867910.322420

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Fencing

* Object type  composite

* Object count 34

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Fencing

* Feature type Simple

* Geometry type Polyline

* Has topology FALSE

* Feature count 34

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Geoprocessing history  ▼►

Process

Process name

Date 2015-11-22 15:16:41

Tool location c:\program files (x86)\arcgis\desktop10.3\ArcToolbox\Toolboxes\Data Management Tools.tbx\CalculateField

Command issued

CalculateField Baseball_Field_Fences TYPE "[TYPE] = CHAINLINK" VB #

Include in lineage when exporting metadata No

Process

Process name

Date 2015-11-22 15:17:18

Tool location c:\program files (x86)\arcgis\desktop10.3\ArcToolbox\Toolboxes\Data Management Tools.tbx\CalculateField

Command issued

CalculateField Baseball_Field_Fences TYPE "[TYPE] = CHAINLINK" VB #

Include in lineage when exporting metadata No

Process

Process name

Date 2015-11-22 15:17:45

Tool location c:\program files (x86)\arcgis\desktop10.3\ArcToolbox\Toolboxes\Data Management Tools.tbx\CalculateField

Command issued

CalculateField Baseball_Field_Fences TYPE "[TYPE] = CHAINLINK" VB #

Include in lineage when exporting metadata No

Hide Geoprocessing history ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Fencing  ▼►

* Type Feature Class

* Row count 34

Definition

Fencein around and within park

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field LEFT_FID  ▼►

* Alias LEFT_FID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Fencing

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Left FID

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field LEFT_FID ▲

Field RIGHT_FID  ▼►

* Alias RIGHT_FID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Fencing

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Right FID

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field RIGHT_FID ▲

Field TYPE  ▼►

* Alias TYPE

* Data type String

* Width 50

* Precision 0

* Scale 0

Field description

Fencing around and within park

Description source

Trimble GeoXT 2005 Series

List of values

Value Fence

Description Fencing

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field TYPE ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Lengt  ▼►

* Alias GNSS_Lengt

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999999

Description GNSS Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Lengt ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Line_ID  ▼►

* Alias Line_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Line Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Line_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Hide Details for object Fencing ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-09 11:21:47

Last modified in ArcGIS for the item 2015-11-30 14:00:33

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:00:33

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Fencing

Entity Type Definition

Fencein around and within park

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label LEFT_FID

Attribute Definition

Fencing

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Left FID

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label RIGHT_FID

Attribute Definition

Fencing

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Right FID

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label TYPE

Attribute Definition

Fencing around and within park

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Fence

Enumerated Domain Value Definition

Fencing

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Lengt

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

GNSS Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Line_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Line Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Field_Boundaries

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Baseball Fields

Summary

Locate and Map the Baseball Fields Boundaries

Description

Baseball Field Boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934906 | East | -97.932755 |
| --- | --- | --- | --- |
| North | 29.876852 | South | 29.874363 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Field_Boundaries

Creation date 2015-10-12 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Field_Boundaries

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934906

* East longitude -97.932755

* North latitude 29.876852

* South latitude 29.874363

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306049.327061

* East longitude 2306722.549796

* South latitude 13867011.256961

* North latitude 13867910.322748

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Field_Boundaries

* Object type  composite

* Object count 5

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Field_Boundaries

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 5

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Field_Boundaries  ▼►

* Type Feature Class

* Row count 5

Definition

Baseball Fields

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Practice  ▼►

* Alias Practice

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Practice Field

Description source

Trimble GeoXT 2005 Series

List of values

Value Practice

Description Practice

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Practice ▲

Field Type  ▼►

* Alias Type

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Practice Fields and Main Baseball Field

Description source

Trimble GeoXT 2005 Series

List of values

Value Main

Description Type of Field

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Type ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-999999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Field_Boundaries ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:51

Last modified in ArcGIS for the item 2015-11-30 14:01:22

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:01:22

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Field_Boundaries

Entity Type Definition

Baseball Fields

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Practice

Attribute Definition

Practice Field

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Practice

Enumerated Domain Value Definition

Practice

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Type

Attribute Definition

Practice Fields and Main Baseball Field

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Main

Enumerated Domain Value Definition

Type of Field

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Georeferenced_points

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Georeference Points

Summary

Location of Georeference Points outside of Park

Description

Georeference Points

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.937857 | East | -97.928502 |
| --- | --- | --- | --- |
| North | 29.879274 | South | 29.873368 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Georeferenced_points

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Georeferenced_points

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.937857

* East longitude -97.928502

* North latitude 29.879274

* South latitude 29.873368

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2305117.157666

* East longitude 2308062.667195

* South latitude 13866661.624786

* North latitude 13868782.557552

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Georeferenced_points

* Object type  point

* Object count 11

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Georeferenced_points

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 11

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Georeferenced_points  ▼►

* Type Feature Class

* Row count 11

Definition

Georefence Points outside of Park

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Comment  ▼►

* Alias Comment

* Data type String

* Width 32

* Precision 0

* Scale 0

Field description

Georeference Points outside of Park

Description source

Trimble GeoXT 2005 Series

List of values

Value Comment

Description Georeferencing Points

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Comment ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Georeferenced_points ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:57

Last modified in ArcGIS for the item 2015-11-30 14:01:42

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:01:42

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Georeferenced_points

Entity Type Definition

Georefence Points outside of Park

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Comment

Attribute Definition

Georeference Points outside of Park

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Comment

Enumerated Domain Value Definition

Georeferencing Points

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Illumination

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Illumination

Summary

Location and mapping of every single illumination pole within the park

Description

Light poles within the park

Credits

There are no credits for this item.

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.935059 | East | -97.932741 |
| --- | --- | --- | --- |
| North | 29.876871 | South | 29.873760 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Illumination

Creation date 2015-10-07 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

ArcGIS item properties

* Name Illumination

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.935059

* East longitude -97.932741

* North latitude 29.876871

* South latitude 29.873760

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-23 00:00:00

Extent in the item's coordinate system

* West longitude 2306002.804517

* East longitude 2306727.028462

* South latitude 13866791.867636

* North latitude 13867916.972997

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Illumination

* Object type  point

* Object count 37

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Illumination

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 37

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Illumination  ▼►

* Type Feature Class

* Row count 37

Definition

Light Poles

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Material  ▼►

* Alias Material

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Metal and wooden Light Poles

Description source

Trimble GeoXT 2005 Series

List of values

Value Metal

Description Type of Material

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Material ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Illumination ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:43

Last modified in ArcGIS for the item 2015-11-30 14:02:18

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:02:18

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Illumination

Entity Type Definition

Light Poles

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Material

Attribute Definition

Metal and wooden Light Poles

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Metal

Enumerated Domain Value Definition

Type of Material

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Impervious_Cover_Total

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Impervious Cover

Summary

location and mapping of impervious cover within park boundaries

Description

impervious cover within park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934850 | East | -97.931699 |
| --- | --- | --- | --- |
| North | 29.876551 | South | 29.874005 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Impervious_Cover_Total

Creation date 2015-11-22 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Impervious_Cover_Total

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934850

* East longitude -97.931699

* North latitude 29.876551

* South latitude 29.874005

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-05 00:00:00

Ending date 2015-11-23 00:00:00

Extent in the item's coordinate system

* West longitude 2306068.088507

* East longitude 2307058.537969

* South latitude 13866884.287399

* North latitude 13867800.899410

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Impervious_Cover_Total

* Object type  composite

* Object count 25

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Impervious_Cover_Total

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 25

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Impervious_Cover_Total  ▼►

* Type Feature Class

* Row count 25

Definition

Impervious cover within park boundaries

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifer

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-99999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Impervious_Cover_Total ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-22 16:14:33

Last modified in ArcGIS for the item 2015-11-30 14:02:37

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:02:37

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Impervious_Cover_Total

Entity Type Definition

Impervious cover within park boundaries

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifer

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Irrigation

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Irrigation

Summary

location of every single sprinkler within the park

Description

sprinklers within park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934905 | East | -97.932356 |
| --- | --- | --- | --- |
| North | 29.876835 | South | 29.874372 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Irrigation

Creation date 2015-10-14 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Irrigation

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934905

* East longitude -97.932356

* North latitude 29.876835

* South latitude 29.874372

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306049.418597

* East longitude 2306849.084320

* South latitude 13867015.608330

* North latitude 13867904.235162

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Irrigation

* Object type  point

* Object count 186

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Irrigation

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 186

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Irrigation  ▼►

* Type Feature Class

* Row count 186

Definition

Sprinklers within Park boundaries

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Type  ▼►

* Alias Type

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Sprinklers

Description source

Trimble GeoXT 2005 Series

List of values

Value Sprinkler Head

Description Sprinkler Type

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Type ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Irrigation ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:46

Last modified in ArcGIS for the item 2015-11-30 14:02:58

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:02:58

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78766

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Irrigation

Entity Type Definition

Sprinklers within Park boundaries

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Type

Attribute Definition

Sprinklers

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Sprinkler Head

Enumerated Domain Value Definition

Sprinkler Type

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Limestone_Blocks

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Limestone Blocks

Summary

Location and mapping of Limestone Blocks within park boundaries

Description

Limestone blocks

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934707 | East | -97.933667 |
| --- | --- | --- | --- |
| North | 29.877188 | South | 29.874332 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Limestone_Blocks

Creation date 2015-10-07 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Limestone_Blocks

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934707

* East longitude -97.933667

* North latitude 29.877188

* South latitude 29.874332

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306112.457841

* East longitude 2306432.418815

* South latitude 13866997.370506

* North latitude 13868033.071846

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Limestone_Blocks

* Object type  point

* Object count 27

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Limestone_Blocks

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 27

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Limestone_Blocks  ▼►

* Type Feature Class

* Row count 27

Definition

Limestone Blocks

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Limestone_Blocks ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:49

Last modified in ArcGIS for the item 2015-11-30 14:03:22

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:03:22

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Limestone_Blocks

Entity Type Definition

Limestone Blocks

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Linear_Drainage_Feature

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Linear Drainage

Summary

location and mapping of a linnear drainage

Description

Linear Drainage

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934544 | East | -97.932279 |
| --- | --- | --- | --- |
| North | 29.875740 | South | 29.873715 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Linear_Drainage_Feature

Creation date 2015-10-16 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Linear_Drainage_Feature

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934544

* East longitude -97.932279

* North latitude 29.875740

* South latitude 29.873715

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306166.146710

* East longitude 2306877.298174

* South latitude 13866777.073374

* North latitude 13867506.810760

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Linear_Drainage_Feature

* Object type  composite

* Object count 4

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Linear_Drainage_Feature

* Feature type Simple

* Geometry type Polyline

* Has topology FALSE

* Feature count 4

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Linear_Drainage_Feature  ▼►

* Type Feature Class

* Row count 4

Definition

Linear Drainage

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field TYPE  ▼►

* Alias TYPE

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Linear Drainage Swell and Linear Drainage

Description source

Trimble GeoXT 2005 Series

List of values

Value Drainage

Description Drainage Type

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field TYPE ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Lengt  ▼►

* Alias GNSS_Lengt

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description GNSS Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Lengt ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Line_ID  ▼►

* Alias Line_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Line Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Line_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Hide Details for object Linear_Drainage_Feature ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:11

Last modified in ArcGIS for the item 2015-11-30 14:03:35

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:03:35

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Linear_Drainage_Feature

Entity Type Definition

Linear Drainage

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label TYPE

Attribute Definition

Linear Drainage Swell and Linear Drainage

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Drainage

Enumerated Domain Value Definition

Drainage Type

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Lengt

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Line_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Line Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Park_Bench

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Bench

Summary

Location of Benches within Park

Description

Benches within Park Boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.932657 | East | -97.931687 |
| --- | --- | --- | --- |
| North | 29.876424 | South | 29.876147 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Park_Bench

Creation date 2015-10-13 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Park_Bench

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.932657

* East longitude -97.931687

* North latitude 29.876424

* South latitude 29.876147

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306756.169480

* East longitude 2307062.668867

* South latitude 13867662.997127

* North latitude 13867761.239385

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Park_Bench

* Object type  point

* Object count 2

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Park_Bench

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 2

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Park_Bench  ▼►

* Type Feature Class

* Row count 2

Definition

Park Benches

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Park_Bench ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:52

Last modified in ArcGIS for the item 2015-11-30 14:03:49

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:03:49

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Park_Bench

Entity Type Definition

Park Benches

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Park_Signage

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Signs

Summary

Location of signs within park boundaries

Description

Signs within and around park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.935091 | East | -97.931818 |
| --- | --- | --- | --- |
| North | 29.877137 | South | 29.873603 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Park_Signage

Creation date 2015-10-14 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Park_Signage

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.935091

* East longitude -97.931818

* North latitude 29.877137

* South latitude 29.873603

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2305993.154602

* East longitude 2307018.608915

* South latitude 13866737.643007

* North latitude 13868013.557778

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Park_Signage

* Object type  point

* Object count 7

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Park_Signage

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 7

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Park_Signage  ▼►

* Type Feature Class

* Row count 7

Definition

Park Signs

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field TYPE  ▼►

* Alias TYPE

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Metal and wooden park signs

Description source

Trimble GeoXT 2005 Series

List of values

Value Metal

Description Material of Signs

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field TYPE ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifer

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Park_Signage ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:34:55

Last modified in ArcGIS for the item 2015-11-30 14:04:03

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:04:03

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San MArcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Park_Signage

Entity Type Definition

Park Signs

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label TYPE

Attribute Definition

Metal and wooden park signs

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Metal

Enumerated Domain Value Definition

Material of Signs

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifer

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Parking_Official

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Parking

Summary

location of official parking areas within park boundaries

Description

official parking spaces within park boundaries

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934627 | East | -97.932384 |
| --- | --- | --- | --- |
| North | 29.875511 | South | 29.874108 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Parking_Official

Creation date 2015-10-15 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Parking_Official

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934627

* East longitude -97.932384

* North latitude 29.875511

* South latitude 29.874108

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306138.300965

* East longitude 2306844.627308

* South latitude 13866919.847695

* North latitude 13867423.377528

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Parking_Official

* Object type  composite

* Object count 2

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Parking_Official

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 2

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Parking_Official  ▼►

* Type Feature Class

* Row count 2

Definition

Official Parking Spaces

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifer

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizonal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifer

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Area  ▼►

* Alias Area

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Unique Area

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-99999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Parking_Official ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:26

Last modified in ArcGIS for the item 2015-11-30 14:04:15

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:04:15

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Omar A. Rivera

Organization's name KMRS Geospatial Consultants

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area TX

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Parking_Official

Entity Type Definition

Official Parking Spaces

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifer

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizonal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifer

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area

Attribute Definition

Unique Area

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Paths

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Park Paths

Summary

Shows the location of Park paths, trails, and sidewalks.

Description

Park paths, trails, and sidewalks.

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.935109 | East | -97.931803 |
| --- | --- | --- | --- |
| North | 29.877604 | South | 29.874432 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Paths

Creation date 2015-10-06 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Paths

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M.

Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Temporal extent

Beginning date 2015-10-06 00:00:00

Ending date 2015-11-30 00:00:00

Extent

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.935109

* East longitude -97.931803

* North latitude 29.877604

* South latitude 29.874432

* Extent contains the resource Yes

Extent in the item's coordinate system

* West longitude 2305984.519566

* East longitude 2307022.066935

* South latitude 13867038.953662

* North latitude 13868183.120757

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 34994581.165044695

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Paths

* Object type  composite

* Object count 12

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Paths

* Feature type Simple

* Geometry type Polyline

* Has topology FALSE

* Feature count 12

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Paths  ▼►

* Type Feature Class

* Row count 12

Definition

Park Paths, Trails, Sidewalks

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Type  ▼►

* Alias Type

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

The paths are represented by a line feature.

Description source

Trimble GeoXT 2005 Series

List of values

Value Path

Description Path

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Type ▲

Field Paved  ▼►

* Alias Paved

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Paths or sidewalks that are impervious or concrete

Description source

Trimble GeoXT 2005 Series

List of values

Value Paved

Description Paths that are Paved

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Paved ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Lengt  ▼►

* Alias GNSS_Lengt

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Global Navigation Satellite System

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99999

Description GNSS Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Lengt ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Line_ID  ▼►

* Alias Line_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Line_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Hide Details for object Paths ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:22

Last modified in ArcGIS for the item 2015-11-30 14:04:27

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:04:27

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Paths

Entity Type Definition

Park Paths, Trails, Sidewalks

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Type

Attribute Definition

The paths are represented by a line feature.

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Path

Enumerated Domain Value Definition

Path

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Paved

Attribute Definition

Paths or sidewalks that are impervious or concrete

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Paved

Enumerated Domain Value Definition

Paths that are Paved

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Lengt

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Line_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Pavilion

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Park Pavilion

Summary

To show location of the Pavilian in Ramon Lucio Park

Description

Park Pavilian ( impervious cover)

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.932289 | East | -97.932187 |
| --- | --- | --- | --- |
| North | 29.876021 | South | 29.875876 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Pavilion

Creation date 2015-10-14 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Pavilion

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.932289

* East longitude -97.932187

* North latitude 29.876021

* South latitude 29.875876

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-14 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306873.516358

* East longitude 2306905.526793

* South latitude 13867563.221408

* North latitude 13867615.570057

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Pavilion

* Object type  composite

* Object count 1

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Pavilion

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 1

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Pavilion  ▼►

* Type Feature Class

* Row count 1

Definition

Park Pavilian

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifer

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Positional Dilution Of Precision

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Height Dilution of Precision

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Corr_Type  ▼►

* Alias Corr_Type

* Data type String

* Width 36

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Corrected data

List of values

Value Corrected

Description Corrected Data

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Corr_Type ▲

Field GPS_Date  ▼►

* Alias GPS_Date

* Data type Date

* Width 8

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Date of data collection

List of values

Value Date

Description Date

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GPS_Date ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Global Navigation Satellite System

List of values

Value 0-999999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Horizontal Precision

List of values

Value 0-99999

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Description source

Trimble GeoXT 2005 Series

Field description

Unique Identifier

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-999999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-999999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Pavilion ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-04 11:58:26

Last modified in ArcGIS for the item 2015-11-30 14:04:40

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:04:40

Item location history

Item copied or moved 2015-11-04 11:58:26

From F:\GIS_Proj_Design_Imp\Project\Export\New folder (2)\Pavilion

To \\TAG316040\E\4427\Project\Export\New folder\Pavilion

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Pavilion

Entity Type Definition

Park Pavilian

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifer

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Corr_Type

Attribute Definition

Corrected data

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Corrected

Enumerated Domain Value Definition

Corrected Data

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GPS_Date

Attribute Definition

Date of data collection

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Date

Enumerated Domain Value Definition

Date

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Definition

Global Navigation Satellite System

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Pervious_ParkingLot

File Geodatabase Feature Class

Thumbnail Not Available

Tags
There are no tags for this item.

Summary

There is no summary for this item.

Description

There is no description for this item.

Credits

There are no credits for this item.

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.932696 | East | -97.932566 |
| --- | --- | --- | --- |
| North | 29.875498 | South | 29.874999 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Pervious_ParkingLot

Presentation formats  * digital map

Hide Citation ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

ArcGIS item properties

* Name Pervious_ParkingLot

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.932696

* East longitude -97.932566

* North latitude 29.875498

* South latitude 29.874999

* Extent contains the resource Yes

Extent in the item's coordinate system

* West longitude 2306747.571336

* East longitude 2306786.921363

* South latitude 13867243.194519

* North latitude 13867424.449202

* Extent contains the resource Yes

Hide Extents ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 34994581.165044695

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Pervious_ParkingLot

* Object type  composite

* Object count 1

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Pervious_ParkingLot

* Feature type Simple

* Geometry type Polygon

* Has topology FALSE

* Feature count 1

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Pervious_ParkingLot  ▼►

* Type Feature Class

* Row count 1

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Roofed  ▼►

* Alias Roofed

* Data type String

* Width 60

* Precision 0

* Scale 0

List of values

Value Roofed

Description Determines if there is a Roof

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Roofed ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field GNSS_Area  ▼►

* Alias GNSS_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

List of values

Value 0-99999

Description GNSS Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field GNSS_Area ▲

Field Avg_Horz_P  ▼►

* Alias Avg_Horz_P

* Data type Double

* Width 8

* Precision 0

* Scale 0

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Avg_Horz_P ▲

Field Area_ID  ▼►

* Alias Area_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Area_ID ▲

Field Shape_Length  ▼►

* Alias Shape_Length

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Length of feature in internal units.

* Description source

Esri

List of values

Value 0-99999

Description Shape Length

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Length ▲

Field Shape_Area  ▼►

* Alias Shape_Area

* Data type Double

* Width 8

* Precision 0

* Scale 0

* Field description

Area of feature in internal units squared.

* Description source

Esri

List of values

Value 0-999999

Description Shape Area

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Positive real numbers that are automatically generated.

Hide Field Shape_Area ▲

Hide Details for object Pervious_ParkingLot ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:35

Last modified in ArcGIS for the item 2015-11-30 14:05:29

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:05:29

Hide Metadata Details ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Pervious_ParkingLot

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Roofed

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Roofed

Enumerated Domain Value Definition

Determines if there is a Roof

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label GNSS_Area

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

GNSS Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Avg_Horz_P

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Area_ID

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Length

Attribute Definition

Length of feature in internal units.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape Length

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape_Area

Attribute Definition

Area of feature in internal units squared.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Positive real numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999999

Enumerated Domain Value Definition

Shape Area

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Picnic_Table_Areas

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Park Picnic Area

Summary

Geographic location of Ramon Lucio's picnic areas

Description

Park Picnic Area

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.932236 | East | -97.931710 |
| --- | --- | --- | --- |
| North | 29.876516 | South | 29.875924 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Picnic_Table_Areas

Creation date 2015-10-13 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Picnic_Table_Areas

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.932236

* East longitude -97.931710

* North latitude 29.876516

* South latitude 29.875924

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-13 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306890.170852

* East longitude 2307055.002872

* South latitude 13867582.056672

* North latitude 13867795.948961

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Picnic_Table_Areas

* Object type  point

* Object count 7

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Picnic_Table_Areas

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 7

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Picnic_Table_Areas  ▼►

* Type Feature Class

* Row count 7

Definition

Park Picnic Areas

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Field Num_tables  ▼►

* Alias Num_tables

* Data type SmallInteger

* Width 2

* Precision 0

* Scale 0

Field description

Number of tables

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-99

Description Number of Tables

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Num_tables ▲

Hide Details for object Picnic_Table_Areas ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-22 14:49:53

Last modified in ArcGIS for the item 2015-11-30 14:05:42

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:05:42

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Picnic_Table_Areas

Entity Type Definition

Park Picnic Areas

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Num_tables

Attribute Definition

Number of tables

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99

Enumerated Domain Value Definition

Number of Tables

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Practice_Backstops

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Practice Backstops

Summary

To show the location of the practice backstops in Ramon Lucio Park.

Description

Practice backstops used for pitchers to practice.

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934938 | East | -97.932919 |
| --- | --- | --- | --- |
| North | 29.876774 | South | 29.874853 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Practice_Backstops

Creation date 2015-10-12 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Practice_Backstops

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934938

* East longitude -97.932919

* North latitude 29.876774

* South latitude 29.874853

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-12 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306037.566586

* East longitude 2306670.922275

* South latitude 13867188.848469

* North latitude 13867881.770312

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Practice_Backstops

* Object type  point

* Object count 14

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Practice_Backstops

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 14

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Practice_Backstops  ▼►

* Type Feature Class

* Row count 14

Definition

Pracrice Backstops

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Practice_Backstops ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:00

Last modified in ArcGIS for the item 2015-11-30 14:05:54

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:05:54

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Practice_Backstops

Entity Type Definition

Pracrice Backstops

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Waste_disposal

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Park Waste

Summary

Locations of the trash cans, moveable and permanent in Ramon Lucio Park

Description

Park Waste

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934768 | East | -97.931740 |
| --- | --- | --- | --- |
| North | 29.876529 | South | 29.874503 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Waste_disposal

Creation date 2015-10-06 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Waste_disposal

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934768

* East longitude -97.931740

* North latitude 29.876529

* South latitude 29.874503

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-06 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306092.463786

* East longitude 2307045.308665

* South latitude 13867065.065252

* North latitude 13867793.146145

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Waste_disposal

* Object type  point

* Object count 36

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Waste_disposal

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 36

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Waste_disposal  ▼►

* Type Feature Class

* Row count 36

Definition

Waste Disposal

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Type  ▼►

* Alias Type

* Data type String

* Width 60

* Precision 0

* Scale 0

Field description

Permanent, moveable

Description source

Trimble GeoXT 2005 Series

List of values

Value Permanent

Description Determines moveablility

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Type ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Waste_disposal ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:03

Last modified in ArcGIS for the item 2015-11-30 14:06:12

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:06:12

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

Country US

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Waste_disposal

Entity Type Definition

Waste Disposal

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Type

Attribute Definition

Permanent, moveable

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value Permanent

Enumerated Domain Value Definition

Determines moveablility

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Water_Fountain

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Water Fountain

Summary

To show the location of the water fountain feature

Description

Water fountain

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.933761 | East | -97.933762 |
| --- | --- | --- | --- |
| North | 29.875183 | South | 29.875184 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:50,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Water_Fountain

Creation date 2015-10-07 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Water_Fountain

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

West longitude -97.933761

* East longitude -97.933762

North latitude 29.875183

* South latitude 29.875184

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-06 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306409.029098

* East longitude 2306409.029098

* South latitude 13867306.956829

* North latitude 13867306.956829

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Water_Fountain

* Object type  point

* Object count 1

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Water_Fountain

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 1

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Water_Fountain  ▼►

* Type Feature Class

* Row count 1

Definition

Water Fountain

Definition source

Trimble GeoXT 2005 Series

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-9999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Water_Fountain ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:06

Last modified in ArcGIS for the item 2015-11-30 14:06:24

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:06:24

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

e-mail address bdm65@txstate.edu

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Water_Fountain

Entity Type Definition

Water Fountain

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-9999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲

# Yellow_Gate

File Geodatabase Feature Class

Thumbnail Not Available

Tags
Yellow Gate

Summary

To show the location of the yellow gate

Description

Yellow gate

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

Use limitations

There are no access and use limitations for this item.

Extent

| West | -97.934654 | East | -97.932761 |
| --- | --- | --- | --- |
| North | 29.875543 | South | 29.874773 |

Scale Range

| Maximum (zoomed in) | 1:5,000 |
| --- | --- |
| Minimum (zoomed out) | 1:150,000,000 |

## ArcGIS Metadata ▼►

## Topics and Keywords  ▼►

* Content type  Downloadable Data

Hide Topics and Keywords ▲

## Citation  ▼►

* Title Yellow_Gate

Creation date 2015-10-13 00:00:00

Publication date 2015-11-30 00:00:00

Presentation formats  * digital map

Hide Citation ▲

## Citation Contacts  ▼►

Responsible party

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Hide Citation Contacts ▲

## Resource Details  ▼►

Dataset languages  * English (UNITED STATES)

Dataset character set  utf8 - 8 bit UCS Transfer Format

Status  completed

Spatial representation type  * vector

* Processing environment Microsoft Windows 7 Version 6.1 (Build 7601) Service Pack 1; Esri ArcGIS 10.3.1.4959

Credits

Carlos Soto; Team Leader, Program Manager, GIS Analyst

Curtis Klinge; Assistant Team Manager, GIS Analyst

Bradly McDonald; GIS Analyst

Omar A. Rivera; GIS Analyst

ArcGIS item properties

* Name Yellow_Gate

* Location file://\\Geoserve\Students\bdm65\4427Project\RamonLucioPark.gdb

* Access protocol Local Area Network

Hide Resource Details ▲

## Extents  ▼►

Extent

Description

Ramon Lucio Park and baseball fields, bounded by Cheatham Street, C.M. Allen Pkwy, Interstate Hwy 35 service road, and the San Marcos River

Geographic extent

Bounding rectangle

Extent type  Extent used for searching

* West longitude -97.934654

* East longitude -97.932761

* North latitude 29.875543

* South latitude 29.874773

* Extent contains the resource Yes

Temporal extent

Beginning date 2015-10-13 00:00:00

Ending date 2015-11-30 00:00:00

Extent in the item's coordinate system

* West longitude 2306127.552299

* East longitude 2306725.163308

* South latitude 13867160.397411

* North latitude 13867435.069105

* Extent contains the resource Yes

Hide Extents ▲

## Resource Maintenance  ▼►

Resource maintenance

Update frequency  annually

Hide Resource Maintenance ▲

## Spatial Reference  ▼►

ArcGIS coordinate system

* Type Projected

* Geographic coordinate reference GCS_North_American_1983

* Projection NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet

* Coordinate reference details

Projected coordinate system

Well-known identifier 102740

X origin -126725700

Y origin -77828800

XY scale 3048.0060960121918

Z origin -100000

Z scale 10000

M origin -100000

M scale 10000

XY tolerance 0.0032808333333333331

Z tolerance 0.001

M tolerance 0.001

High precision true

Latest well-known identifier 2278

Well-known text PROJCS["NAD_1983_StatePlane_Texas_South_Central_FIPS_4204_Feet",GEOGCS["GCS_North_American_1983",DATUM["D_North_American_1983",SPHEROID["GRS_1980",6378137.0,298.257222101]],PRIMEM["Greenwich",0.0],UNIT["Degree",0.0174532925199433]],PROJECTION["Lambert_Conformal_Conic"],PARAMETER["False_Easting",1968500.0],PARAMETER["False_Northing",13123333.33333333],PARAMETER["Central_Meridian",-99.0],PARAMETER["Standard_Parallel_1",28.38333333333333],PARAMETER["Standard_Parallel_2",30.28333333333334],PARAMETER["Latitude_Of_Origin",27.83333333333333],UNIT["Foot_US",0.3048006096012192],AUTHORITY["EPSG",2278]]

Reference system identifier

* Value 2278

* Codespace EPSG

* Version 8.6.2

Hide Spatial Reference ▲

## Spatial Data Properties  ▼►

Vector  ▼►

* Level of topology for this dataset  geometry only

Geometric objects

Feature class name Yellow_Gate

* Object type  point

* Object count 2

Hide Vector ▲

ArcGIS Feature Class Properties  ▼►

Feature class name Yellow_Gate

* Feature type Simple

* Geometry type Point

* Has topology FALSE

* Feature count 2

* Spatial index TRUE

* Linear referencing FALSE

Hide ArcGIS Feature Class Properties ▲

Hide Spatial Data Properties ▲

## Distribution  ▼►

Distribution format

* Name File Geodatabase Feature Class

Hide Distribution ▲

## Fields  ▼►

Details for object Yellow_Gate  ▼►

* Type Feature Class

* Row count 2

Definition source

Trimble GeoXT 2005 Series

Definition

Yellow Gate

Field OBJECTID  ▼►

* Alias OBJECTID

* Data type OID

* Width 4

* Precision 0

* Scale 0

* Field description

Internal feature number.

* Description source

Esri

List of values

Value 0-999

Description Unique Identifeier

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Sequential unique whole numbers that are automatically generated.

Hide Field OBJECTID ▲

Field Shape  ▼►

* Alias Shape

* Data type Geometry

* Width 0

* Precision 0

* Scale 0

* Field description

Feature geometry.

* Description source

Esri

List of values

Value 0-99999

Description Shape

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

* Description of values

Coordinates defining the features.

Hide Field Shape ▲

Field Max_PDOP  ▼►

* Alias Max_PDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Positional Dilution Of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description PDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_PDOP ▲

Field Max_HDOP  ▼►

* Alias Max_HDOP

* Data type Single

* Width 4

* Precision 0

* Scale 0

Field description

Height Dilution of Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description HDOP

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Max_HDOP ▲

Field Horz_Prec  ▼►

* Alias Horz_Prec

* Data type Double

* Width 8

* Precision 0

* Scale 0

Field description

Horizontal Precision

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-20

Description Horizontal Precision

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Horz_Prec ▲

Field Point_ID  ▼►

* Alias Point_ID

* Data type Integer

* Width 4

* Precision 0

* Scale 0

Field description

Unique Identifier

Description source

Trimble GeoXT 2005 Series

List of values

Value 0-999

Description Unique Identifer

Enumerated domain value definition source Trimble GeoXT 2005 Series Unit

Hide Field Point_ID ▲

Hide Details for object Yellow_Gate ▲

Hide Fields ▲

## Metadata Details  ▼►

* Metadata language English (UNITED STATES)

Scope of the data described by the metadata  * dataset

Scope name  * dataset

* Last update 2015-11-30

ArcGIS metadata properties

Metadata format ArcGIS 1.0

Metadata style FGDC CSDGM Metadata

Standard or profile used to edit metadata FGDC

Created in ArcGIS for the item 2015-11-23 11:35:09

Last modified in ArcGIS for the item 2015-11-30 14:06:34

Automatic updates

Have been performed Yes

Last update 2015-11-30 14:06:34

Hide Metadata Details ▲

## Metadata Contacts  ▼►

Metadata contact

Individual's name Bradly McDonald

Organization's name KMRS Geospatial Consultants

Contact's position Geospatial Consultant

Contact's role  originator

Contact information  ▼►

Phone

Voice 512-245-2111

Address

Type postal

Delivery point 601 University Dr

City San Marcos

Administrative area Texas

Postal code 78666

Hide Contact information ▲

Hide Metadata Contacts ▲

## FGDC Metadata (read-only) ▼►

## Entities and Attributes  ▼►

Detailed Description

Entity Type

Entity Type Label Yellow_Gate

Entity Type Definition

Yellow Gate

Entity Type Definition Source Trimble GeoXT 2005 Series

Attribute

Attribute Label OBJECTID

Attribute Definition

Internal feature number.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Sequential unique whole numbers that are automatically generated.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifeier

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Shape

Attribute Definition

Feature geometry.

Attribute Definition Source Esri

Attribute Domain Values

Unrepresentable Domain

Coordinates defining the features.

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-99999

Enumerated Domain Value Definition

Shape

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_PDOP

Attribute Definition

Positional Dilution Of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

PDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Max_HDOP

Attribute Definition

Height Dilution of Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

HDOP

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Horz_Prec

Attribute Definition

Horizontal Precision

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-20

Enumerated Domain Value Definition

Horizontal Precision

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Attribute

Attribute Label Point_ID

Attribute Definition

Unique Identifier

Attribute Definition Source Trimble GeoXT 2005 Series

Attribute Domain Values

Enumerated Domain

Enumerated Domain Value 0-999

Enumerated Domain Value Definition

Unique Identifer

Enumerated Domain Value Definition Source

Trimble GeoXT 2005 Series Unit

Hide Entities and Attributes ▲